`timescale 1ns / 1ps

module Ploca(

input wire CLK,

inout wire PS2_DATA, PS2_CLK,

input wire RESET,

input wire SW0,

output VGA_HSYNC, VGA_VSYNC,

output VGA_G, VGA_B, VGA_R,

output [7:0] led_state

);

// Clock divider za 25 MHz (od 50 MHz)

reg clk25 = 0;

always @(posedge CLK) begin

clk25 <= ~clk25;

end

// VGA 640x480@60Hz timing parametri

parameter h_display = 640;

parameter h_frontporch = 16;

parameter h_syncwidth = 96;

parameter h_backporch = 48;

parameter h_total = 800;

parameter v_display = 480;

parameter v_frontporch = 10;

parameter v_syncwidth = 2;

parameter v_backporch = 33;

parameter v_total = 525;

// VGA brojaèi i signali

reg [9:0] h_counter = 0;

reg [9:0] v_counter = 0;

reg hsync, vsync;

// Grid parametri

localparam CELL_WIDTH = 16;

localparam CELL_HEIGHT = 16;

localparam GRID_COLS = 8;

localparam GRID_ROWS = 8;

localparam GRID_WIDTH = GRID_COLS * CELL_WIDTH;

localparam GRID_HEIGHT = GRID_ROWS * CELL_HEIGHT;

localparam GRID_X_OFFSET = (640 - GRID_WIDTH) / 2;

localparam GRID_Y_OFFSET = (480 - GRID_HEIGHT) / 2;

// Signali za crtanje (mora biti reg jer se koriste u always bloku)

reg grid;

// Miš: signali

wire [8:0] mouse_x, mouse_y;

wire [2:0] mouse_btn;

wire mouse_done_tick;

// Instanciranje modula za miš

mouse mouse_unit (

.clk(CLK),

.reset(RESET),

.ps2d(PS2_DATA),

.ps2c(PS2_CLK),

.xm(mouse_x),

.ym(mouse_y),

.btnm(mouse_btn),

.m_done_tick(mouse_done_tick)

);

// Registarske pozicije kursora

reg [9:0] cursor_x = 320;

reg [9:0] cursor_y = 240;

// Podešavanje osjetljivosti

wire signed [9:0] scaled_mouse_x = $signed(mouse_x) >>> 1;

wire signed [9:0] scaled_mouse_y = $signed(mouse_y) >>> 1;

// Ažuriranje pozicije kursora

always @(posedge CLK or posedge RESET) begin

if (RESET) begin

cursor_x <= 320;

cursor_y <= 240;

end else if (mouse_done_tick) begin

if ($signed({1'b0, cursor_x}) + scaled_mouse_x > 639) cursor_x <= 639;

else if ($signed({1'b0, cursor_x}) + scaled_mouse_x < 0) cursor_x <= 0;

else cursor_x <= cursor_x + scaled_mouse_x;

if ($signed({1'b0, cursor_y}) - scaled_mouse_y > 479) cursor_y <= 479;

else if ($signed({1'b0, cursor_y}) - scaled_mouse_y < 0) cursor_y <= 0;

else cursor_y <= cursor_y - scaled_mouse_y;

end

end

// ROM za strelicu

reg [7:0] arrow_rom [0:7];

initial begin

arrow_rom[0] = 8'b0000_0001;

arrow_rom[1] = 8'b0000_0011;

arrow_rom[2] = 8'b0000_0111;

arrow_rom[3] = 8'b0000_1111;

arrow_rom[4] = 8'b0000_0111;

arrow_rom[5] = 8'b0000_0011;

arrow_rom[6] = 8'b0000_0001;

arrow_rom[7] = 8'b0000_0000;

end

// Signali za crtanje

wire [9:0] sprite_x = h_counter - cursor_x;

wire [9:0] sprite_y = v_counter - cursor_y;

wire sprite_area = (sprite_x < 8) && (sprite_y < 8) && (sprite_x >= 0) && (sprite_y >= 0);

wire [2:0] sprite_x_rel = sprite_x[2:0];

wire [2:0] sprite_y_rel = sprite_y[2:0];

wire sprite_bit = arrow_rom[sprite_y_rel][sprite_x_rel];

reg [3:0] susjedniBr;

// Glavni crtaèki logika

always @(posedge clk25 or posedge RESET) begin

if (RESET) begin

h_counter <= 0;

v_counter <= 0;

hsync <= 1;

vsync <= 1;

grid <= 0;

end else if (SW0) begin

h_counter <= (h_counter == h_total - 1) ? 0 : h_counter + 1;

h_counter <= (h_counter == h_total - 1) ? 0 : h_counter + 1;

if (h_counter == h_total - 1)

v_counter <= (v_counter == v_total - 1) ? 0 : v_counter + 1;

// Sync signali

hsync <= ~((h_counter >= h_display + h_frontporch) &&

(h_counter < h_display + h_frontporch + h_syncwidth));

vsync <= ~((v_counter >= v_display + v_frontporch) &&

(v_counter < v_display + v_frontporch + v_syncwidth));

// Grid logika

grid <= (h_counter >= GRID_X_OFFSET && h_counter <= GRID_X_OFFSET + GRID_WIDTH + 1 &&

v_counter >= GRID_Y_OFFSET && v_counter <= GRID_Y_OFFSET + GRID_HEIGHT + 1) &&

(((h_counter - GRID_X_OFFSET) % CELL_WIDTH < 2) ||

((v_counter - GRID_Y_OFFSET) % CELL_HEIGHT < 2));

end

end

wire cursor_in_grid = (cursor_x >= GRID_X_OFFSET) && (cursor_x < GRID_X_OFFSET + GRID_WIDTH) &&

(cursor_y >= GRID_Y_OFFSET) && (cursor_y < GRID_Y_OFFSET + GRID_HEIGHT);

// Sigurno izraèunavanje pozicije samo ako je kursor u gridu

wire [3:0] grid_col = cursor_in_grid ? ((cursor_x - GRID_X_OFFSET) / CELL_WIDTH) : 4'hF;

wire [3:0] grid_row = cursor_in_grid ? ((cursor_y - GRID_Y_OFFSET) / CELL_HEIGHT) : 4'hF;

////////////////////////////////////////

reg [6:0] seed_counter = 0;

always @(posedge CLK) begin

seed_counter <= seed_counter + 1;

end

///////////////////////////////////////////

reg [3:0] polje [0:63];

reg [6:0] init_counter = 0;

reg [3:0] countp = 0;  // 2-bitni brojaè za 0-1-2 ciklus

reg switch = 0;

wire in_grid_area = (h_counter >= GRID_X_OFFSET) && (h_counter <= GRID_X_OFFSET + GRID_WIDTH +2) && (v_counter >= GRID_Y_OFFSET) && (v_counter <= GRID_Y_OFFSET + GRID_HEIGHT + 1);

wire [6:0] first_click_index;

wire first_click_done;

wire [6:0] clicked_cell;

wire [63:0] grid_state; // 100-bitni vektor za stanje polja

wire [63:0] grid_state2;

grid_click click_unit (

.clk(CLK),

.reset(RESET),

.grid_col(grid_col),

.grid_row(grid_row),

.mouse_click(mouse_btn[0]),

// .led_state(led_state),

.grid_state(grid_state),

.first_click_index(first_click_index),

.first_click_done(first_click_done)

);

grid_click_right click_unit_right (

.clk(CLK),

.reset(RESET),

.grid_col(grid_col),

.grid_row(grid_row),

.mouse_click(mouse_btn[1]),

.grid_state(grid_state2)

);

localparam CELL_CLOSED = 2'b00;

localparam CELL_OPENED = 2'b01;

localparam CELL_FLAGGED = 2'b10;

reg [1:0] cell_state [0:63];  // Stanje za svaku æeliju

// Logika igre

// Sadržaj æelije (0-8: broj mina, 9: mina)

reg game_over = 0, game_win = 0;

reg [6:0] opened_count = 0;   // Broj otvorenih æelija bez mina

integer k;

// FSM za upravljanje stanjima æelija

// 1. Sinkronizacija ulaza za cursor poziciju

reg [3:0] grid_col_sync, grid_row_sync;

always @(posedge CLK) begin

grid_col_sync <= grid_col;

grid_row_sync <= grid_row;

end

// 2. Kombinacijski indeks (sinkronizirano)

wire [6:0] current_idx = grid_row_sync * 8 + grid_col_sync;

// 3. Registar za praæenje otvorenih æelija

// 4. Ispravljeni FSM za stanja æelija

reg [1:0] zero_opening [0:63]; // 0 = neaktivno, 1 = èekanje, 2 = otvorena nula, 3 = otvoren broj

reg processing_zero;

reg [6:0] d;

reg [6:0] h;

reg found_one;

reg [6:0] polje_index;

reg[63:0] opened_history;

reg [63:0] check = 0;

integer o;

reg[4:0] flag_count;

////////////////////////////////////////////

always @(posedge CLK) begin //1

if (RESET) begin //2

// Reset kontrolnih signala

game_over       <= 0;

game_win        <= 0;

opened_count    <= 0;

processing_zero <= 0;

flag_count <= 0;

d               = 0;

// Reset stanja svih æelija

for (h = 0; h < 64; h = h + 1) begin //3

cell_state[h]      <= CELL_CLOSED;

zero_opening[h]    <= 0;

opened_history[h]  <= 0;

end //3

end //2

else if (!game_over && !game_win && !processing_zero) begin //2

// Detekcija dogaðaja miša

if (mouse_done_tick && cursor_in_grid) begin //3

// DESNI KLIK - flag logika

if (mouse_btn[1] && !mouse_btn[0]) begin //4

if (cell_state[current_idx] == CELL_CLOSED) begin //5

cell_state[current_idx] <= CELL_FLAGGED;

flag_count <= flag_count + 1;

end //5

else if (cell_state[current_idx] == CELL_FLAGGED) begin //5

cell_state[current_idx] <= CELL_CLOSED;

flag_count <= flag_count - 1;

end //5

end //4

// LIJEVI KLIK - otvaranje æelije

else if (mouse_btn[0] && !mouse_btn[1]) begin //4

if (cell_state[current_idx] == CELL_CLOSED) begin //5

// Oznaci æeliju kao otvorenu

cell_state[current_idx] <= CELL_OPENED;

// Provjera je li mina

if (polje[current_idx] == 9) begin //6

game_over <= 1; // GAME OVER

end //6

// Brojanje samo ako nije mina i nije veæ brojana

else if (!opened_history[current_idx]) begin //6

opened_history[current_idx] <= 1'b1; // Oznaèi kao brojanu

opened_count <= opened_count + 1;    // Poveæaj brojaè

// Provjera pobjede (90 otvorenih æelija)

if (opened_count == 53) begin //7

game_win <= 1; // POBJEDA

end //7

end //6

// Pokreni flood fill ako je nula

if (polje[current_idx] == 0 && first_click_done) begin //6

zero_opening[current_idx] <= 1;

processing_zero <= 1;

d = 0;

end //6

end //5

end //4

end //3

end //2

// Flood fill logika

else if (processing_zero) begin //2

if (d < 64) begin //3

if (zero_opening[d] == 1) begin //4

if (polje[d] == 0) begin //5

zero_opening[d] <= 2;

cell_state[d]   <= CELL_OPENED;

if (!opened_history[d]) begin //6

opened_history[d] <= 1;

opened_count <= opened_count + 1;

if (opened_count == 53)

game_win <= 1;

end //6

/*      // Gore-lijevo

if (d > 7 && d[2:0] != 0) begin

if (zero_opening[d-9] == 0)

zero_opening[d - 9] <= 1;

end

// Gore

if (d >= 8 ) begin

if (zero_opening[d - 8] == 0)

zero_opening[d - 8] <= 1;

end

// Gore-desno

if (d > 7 && d[2:0] != 7 ) begin

if (zero_opening[d - 7] == 0)

zero_opening[d - 7] <= 1;

end

// Lijevo

if (d[2:0] != 0) begin

if (zero_opening[d - 1] == 0)

zero_opening[d - 1] <= 1;

end

// Desno

if (d[2:0] != 7)

zero_opening[d + 1] <= 1;

// Dolje-lijevo

if (d < 56 && d[2:0] != 0 )

zero_opening[d + 7] <= 1;

// Dolje

if (d < 56)

zero_opening[d + 8] <= 1;

// Dolje-desno

if (d < 55 && d[2:0] != 7)

zero_opening[d + 9] <= 1;

*/

d = -1;

end //5

else if (polje[d] != 9) begin //5

zero_opening[d] <= 3;

cell_state[d]   <= CELL_OPENED;

if (!opened_history[d]) begin //6

opened_history[d] <= 1;

opened_count <= opened_count + 1;

if (opened_count == 53)

game_win <= 1;

end //6

end //5

end //4

d = d + 1;

end //3

else begin //3

processing_zero <= 0;

d = 0;

end //3

end //2

else if (game_over) begin //2

for (o = 0; o < 64; o = o + 1) begin //3

if (polje[o] == 9)

cell_state[o] <= CELL_OPENED;

end //3

end //2

end //1

// Odreðivanje trenutne æelije

wire [3:0] current_col = ((h_counter - GRID_X_OFFSET - 2) / CELL_WIDTH);

wire [3:0] current_row = ((v_counter - GRID_Y_OFFSET - 1) / CELL_HEIGHT);

parameter PLACE_MINES = 2'd0;

parameter COUNT_MINES = 2'd1;

parameter DONE        = 2'd2;

reg [1:0] state;

reg [63:0] mines;

reg [6:0] lfsr;

reg [3:0] mine_counter;

// Counters for grid traversal

reg [3:0] mine_count;

reg [3:0] row, col;

wire left_edge, right_edge, top_edge, bottom_edge;

assign left_edge   = (col == 0);

assign right_edge  = (col == 7);

assign top_edge    = (row == 0);

assign bottom_edge = (row == 7);

reg grid_done = 0;

integer i;

always @(posedge CLK) begin

if (RESET) begin

state <= PLACE_MINES;

mines <= 0;

lfsr <= 7'b1010101;

mine_counter <= 0;

polje_index <= 0;

row <= 0;

col <= 0;

grid_done <= 0;

for (i = 0; i < 64; i = i+1)

polje[i] <= 0;

end else begin

case (state)

//  PLACE MINES

PLACE_MINES: begin

if (first_click_done && mine_counter < 10) begin

// LFSR for pseudo-random position

lfsr <= {lfsr[5:0], lfsr[6] ^ lfsr[5]} ^ seed_counter;

if (lfsr < 64 && lfsr != first_click_index && !mines[lfsr]) begin

mines[lfsr] <= 1'b1;

mine_counter <= mine_counter + 1;

polje[lfsr] <= 9; // 9 means mine

end

end

if (mine_counter == 10) begin

// Start counting mines

state <= COUNT_MINES;

polje_index <= 0;

row <= 0;

col <= 0;

end

end

//  COUNT MINES

COUNT_MINES: begin

// Only count if this cell is not a mine

if (polje[polje_index] != 9) begin

mine_count = 0;

// Check all 8 neighbors

// North-West

if (!top_edge && !left_edge)

if (polje[polje_index-9] == 9) mine_count = mine_count + 1;

// North

if (!top_edge)

if (polje[polje_index-8] == 9) mine_count = mine_count + 1;

// North-East

if (!top_edge && !right_edge)

if (polje[polje_index-7] == 9) mine_count = mine_count + 1;

// West

if (!left_edge)

if (polje[polje_index-1] == 9) mine_count = mine_count + 1;

// East

if (!right_edge)

if (polje[polje_index+1] == 9) mine_count = mine_count + 1;

// South-West

if (!bottom_edge && !left_edge)

if (polje[polje_index+7] == 9) mine_count = mine_count + 1;

// South

if (!bottom_edge)

if (polje[polje_index+8] == 9) mine_count = mine_count + 1;

// South-East

if (!bottom_edge && !right_edge)

if (polje[polje_index+9] == 9) mine_count = mine_count + 1;

polje[polje_index] <= mine_count;

end

// Move to next cell

if (col == 7) begin

col <= 0;

if (row == 7) begin

// All cells processed

state <= DONE;

grid_done <= 1;

end else begin

row <= row + 1;

end

end else begin

col <= col + 1;

end

polje_index <= polje_index + 1;

end

// ========== DONE ==========

DONE: begin

end

endcase

end

end

reg [7:0] jedan [0:7];

initial begin

jedan[0] = 8'b00110000;

jedan[1] = 8'b00111000;

jedan[2] = 8'b00110100;

jedan[3] = 8'b00110000;

jedan[4] = 8'b00110000;

jedan[5] = 8'b00110000;

jedan[6] = 8'b00110000;

jedan[7] = 8'b01111000;

end

reg [7:0] mine_rom [0:7];

initial begin

mine_rom[0] = 8'b00100100;

mine_rom[1] = 8'b00011000;

mine_rom[2] = 8'b10111101;

mine_rom[3] = 8'b01111110;

mine_rom[4] = 8'b01111110;

mine_rom[5] = 8'b10111101;

mine_rom[6] = 8'b00011000;

mine_rom[7] = 8'b00100100;

end

reg [7:0] nula [0:7];

initial begin

nula[0] = 8'b11111111;

nula[1] = 8'b11111111;

nula[2] = 8'b11111111;

nula[3] = 8'b11111111;

nula[4] = 8'b11111111;

nula[5] = 8'b11111111;

nula[6] = 8'b11111111;

nula[7] = 8'b11111111;

end

reg [7:0]dva[0:7];

initial begin

dva[0] = 8'b01111110;

dva[1] = 8'b11000011;

dva[2] = 8'b11000000;

dva[3] = 8'b01110000;

dva[4] = 8'b00011100;

dva[5] = 8'b00001110;

dva[6] = 8'b00000011;

dva[7] = 8'b11111111;

end

reg [7:0] three[0:7];

initial begin

three[0] = 8'b01111110;

three[1] = 8'b11000011;

three[2] = 8'b11100000;

three[3] = 8'b01111000;

three[4] = 8'b01111000;

three[5] = 8'b11100000;

three[6] = 8'b11000011;

three[7] = 8'b01111110;

end

reg [7:0] four[0:7];

initial begin

four[0] = 8'b00011000;

four[1] = 8'b00011100;

four[2] = 8'b00011010;

four[3] = 8'b00111111;

four[4] = 8'b00011000;

four[5] = 8'b00011000;

four[6] = 8'b00011000;

four[7] = 8'b00011000;

end

reg [7:0] five[0:7];

initial begin

five[0] = 8'b01111111;

five[1] = 8'b00000011;

five[2] = 8'b00000011;

five[3] = 8'b00111111;

five[4] = 8'b01100000;

five[5] = 8'b01100000;

five[6] = 8'b01100011;

five[7] = 8'b00111110;

end

reg [7:0] six[0:7];

initial begin

six[0] = 8'b01111100;

six[1] = 8'b11000110;

six[2] = 8'b00000011;

six[3] = 8'b01111111;

six[4] = 8'b11000011;

six[5] = 8'b11000011;

six[6] = 8'b11000110;

six[7] = 8'b01111100;

end

reg [7:0] seven[0:7];

initial begin

seven[0] = 8'b11111111;

seven[1] = 8'b11000000;

seven[2] = 8'b01100000;

seven[3] = 8'b00110000;

seven[4] = 8'b00011000;

seven[5] = 8'b00001100;

seven[6] = 8'b00000110;

seven[7] = 8'b00000011;

end

reg [7:0] eight[0:7];

initial begin

eight[0] = 8'b00111100;

eight[1] = 8'b01100110;

eight[2] = 8'b01100110;

eight[3] = 8'b00111100;

eight[4] = 8'b01100110;

eight[5] = 8'b01100110;

eight[6] = 8'b01100110;

eight[7] = 8'b00111100;

end

reg [7:0] nine[0:7];

initial begin

nine[0] = 8'b01111110;

nine[1] = 8'b11000011;

nine[2] = 8'b11000011;

nine[3] = 8'b11000011;

nine[4] = 8'b11111110;

nine[5] = 8'b11000000;

nine[6] = 8'b01100000;

nine[7] = 8'b00111110;

end

reg [7:0] flag[0:7];

initial begin

flag[0] = 8'b00111000;

flag[1] = 8'b01111000;

flag[2] = 8'b11111000;

flag[3] = 8'b00011000;

flag[4] = 8'b00011000;

flag[5] = 8'b00011000;

flag[6] = 8'b00011000;

flag[7] = 8'b01111110;

end

reg [15:0] flag_16x16 [0:15];

initial begin

// Originalni redovi (8x8) se dupliraju u 16x16

// Svaki piksel se pretvara u 2x2 blok

flag_16x16[0]  = 16'b0000_0011_1000_0000; // Original: 00111000

flag_16x16[1]  = 16'b0000_0111_1000_0000;

flag_16x16[2]  = 16'b0000_1111_1000_0000;

flag_16x16[3]  = 16'b0001_1111_1000_0000;

flag_16x16[4]  = 16'b0011_1111_1000_0000;

flag_16x16[5]  = 16'b0111_1111_1000_0000;

flag_16x16[6]  = 16'b1111_1111_1000_0000;

flag_16x16[7]  = 16'b0000_0011_1000_0000;

flag_16x16[8]  = 16'b0000_0011_1000_0000;

flag_16x16[9]  = 16'b0000_0011_1000_0000;

flag_16x16[10] = 16'b0000_0011_1000_0000;

flag_16x16[11] = 16'b0000_0011_1000_0000;

flag_16x16[12] = 16'b0000_0011_1000_0000;

flag_16x16[13] = 16'b0000_0011_1000_0000;

flag_16x16[14] = 16'b0111_1111_1111_1110;

flag_16x16[15] = 16'b0111_1111_1111_1110;

end

reg [7:0] nula_num[0:7];

initial begin

nula_num[0] = 8'b00111100;

nula_num[1] = 8'b01100110;

nula_num[2] = 8'b01100110;

nula_num[3] = 8'b01100110;

nula_num[4] = 8'b01100110;

nula_num[5] = 8'b01100110;

nula_num[6] = 8'b01100110;

nula_num[7] = 8'b00111100;

end

reg [7:0] line[0:7];

initial begin

line[0] = 8'b00000000;

line[1] = 8'b00000000;

line[2] = 8'b00000000;

line[3] = 8'b11111111;

line[4] = 8'b11111111;

line[5] = 8'b00000000;

line[6] = 8'b00000000;

line[7] = 8'b00000000;

end

reg [15:0] smiley_rom [0:15];

initial begin

// Bijeli smajli sa crnim pozadinom

smiley_rom[0]  = 16'b0000001111000000;

smiley_rom[1]  = 16'b0000111111110000;

smiley_rom[2]  = 16'b0011111111111100;

smiley_rom[3]  = 16'b0011111111111100;

smiley_rom[4]  = 16'b0110101111010110;

smiley_rom[5]  = 16'b0111011111101110;

smiley_rom[6]  = 16'b1110101111010111;

smiley_rom[7]  = 16'b1111111111111111;

smiley_rom[8]  = 16'b1111111111111111;

smiley_rom[9]  = 16'b1111100000011111;

smiley_rom[10] = 16'b0111011111101110;

smiley_rom[11] = 16'b0110111111110110;

smiley_rom[12] = 16'b0011111111111100;

smiley_rom[13] = 16'b0011111111111100;

smiley_rom[14] = 16'b0000111111110000;

smiley_rom[15] = 16'b0000001111000000;

end

reg [15:0] smiley_rom2 [0:15];

initial begin

// Bijeli smajli sa crnim pozadinom

smiley_rom2[0]  = 16'b0000001111000000;

smiley_rom2[1]  = 16'b0000111111110000;

smiley_rom2[2]  = 16'b0011111111111100;

smiley_rom2[3]  = 16'b0011111111111100;

smiley_rom2[4]  = 16'b0111001111001110;

smiley_rom2[5]  = 16'b0111001111001110;

smiley_rom2[6]  = 16'b1111111111111111;

smiley_rom2[7]  = 16'b1111111111111111;

smiley_rom2[8]  = 16'b1101111111111011;

smiley_rom2[9]  = 16'b1110111111110111;

smiley_rom2[10] = 16'b0111011111101110;

smiley_rom2[11] = 16'b0111100000011110;

smiley_rom2[12] = 16'b0011111111111100;

smiley_rom2[13] = 16'b0011111111111100;

smiley_rom2[14] = 16'b0000111111110000;

smiley_rom2[15] = 16'b0000001111000000;

end

parameter SMILEY_CENTER_X = 322;

parameter SMILEY_CENTER_Y = 160;

parameter SMILEY_SIZE = 16;  // 16x16 piksela

parameter FlagNum_X = 299;

parameter FlagNum_Y = 160;

parameter FlagNum_SIZE = 16;

wire [9:0] Flag_x = FlagNum_X - (FlagNum_SIZE / 2);

wire [9:0] Flag_y = FlagNum_Y - (FlagNum_SIZE / 2);

wire FlagNum_area = (h_counter >= Flag_x) &&

(h_counter < Flag_x + FlagNum_SIZE) &&

(v_counter >= Flag_y) &&

(v_counter < Flag_y + FlagNum_SIZE);

wire [3:0] FlagNum_rel_x = h_counter - Flag_x;

wire [3:0] FlagNum_rel_y = v_counter - Flag_y;

wire FlagNum_pixel = flag_16x16[FlagNum_rel_y][15 - FlagNum_rel_x];

wire show_FlagNum = FlagNum_area && FlagNum_pixel;

// Izraèunaj gornju lijevu poziciju (centriranje)

wire [9:0] smiley_x = SMILEY_CENTER_X - (SMILEY_SIZE / 2);  // 320 - 8 = 312

wire [9:0] smiley_y = SMILEY_CENTER_Y - (SMILEY_SIZE / 2);  // 70 - 8 = 62

// 3. PROVJERA DA LI JE PIKSEL UNUTAR SPRITEA

wire smiley_area = (h_counter >= smiley_x) &&

(h_counter < smiley_x + SMILEY_SIZE) &&

(v_counter >= smiley_y) &&

(v_counter < smiley_y + SMILEY_SIZE);

// 4. IZRAÈUNAJ RELATIVNE KOORDINATE UNUTAR SPRITEA

wire [3:0] smiley_rel_x = h_counter - smiley_x;  // 0-15

wire [3:0] smiley_rel_y = v_counter - smiley_y;  // 0-15

// 5. PROÈITAJ PIKSEL IZ ROM-A (obrnuti redosljed zbog MSB)

wire smiley_pixel = smiley_rom[smiley_rel_y][15 - smiley_rel_x];

wire smiley2_pixel = smiley_rom2[smiley_rel_y][15 - smiley_rel_x];

// 6. PRIKAZUJ SPRITE SAMO KADA JE IZABRAN (npr. game_over)

wire smiley_active = game_over; // Ili bilo koji drugi uslov

wire smiley2_active = game_win;

// 7. KOMBINOVANI SIGNAL ZA PRIKAZ

wire show_smiley = smiley_area && smiley_active && smiley_pixel;

wire show_smiley2 = smiley_area && smiley2_active && smiley2_pixel;

/*wire [9:0] smiley_x = 320;

wire [9:0] smiley_y = 70;

wire [2:0] smiley_width = 32;

wire [2:0] smiley_height = 32;

wire [4:0] smiley_pixel_x = h_counter - smiley_x;

wire [4:0] smiley_pixel_y = v_counter - smiley_y;

wire smiley_sprite_area =(h_counter >= smiley_x) && (h_counter < (smiley_x + smiley_width)) &&

(v_counter >= smiley_y) && (v_counter < (smiley_y + smiley_height));

wire [2:0] sprites_x_index = smiley_pixel_x[4:2];  // Gornja 3 bita (dijeljenje s 4)

wire [2:0] sprites_y_index = smiley_pixel_y[4:2];

wire [7:0] selected_sprite_row =

(game_win) ? happy_face[sprites_y_index] :

(game_over) ? sad_face[sprites_y_index] : 8'd0;

wire sprites_pixel = selected_sprite_row[7 - sprites_x_index];

wire smiley_pixel_active = smiley_sprite_area && (game_win || game_over) && sprites_pixel;

assign led_state = smiley_pixel_active;

*/

//////////////////////////////////

wire [3:0] cell_pixel_x = (h_counter - GRID_X_OFFSET -2) % CELL_WIDTH;

wire [3:0] cell_pixel_y = (v_counter - GRID_Y_OFFSET -1) % CELL_HEIGHT;

// Koordinate za 8x8 sprite centriran u 16x16 æeliji

wire [2:0] cell_sprite_x;

wire [2:0] cell_sprite_y;

assign cell_sprite_x = (cell_pixel_x >= 4 && cell_pixel_x < 12) ? (cell_pixel_x - 4) : 3'b0;

assign cell_sprite_y = (cell_pixel_y >= 4 && cell_pixel_y < 12) ? (cell_pixel_y - 4) : 3'b0;

// Da li smo unutar centra æelije (gdje crtamo sprite)

wire cell_sprite_area = (cell_pixel_x >= 4) && (cell_pixel_x < 12) &&

(cell_pixel_y >= 4) && (cell_pixel_y < 12);

// Kombinirana logika za odabir sprite-a

wire [6:0] current_index = current_row * 8 + current_col;

wire [1:0] current_cell_state = cell_state[current_index];

wire cell_jedan = in_grid_area && (current_col < 8) && (current_row < 8) &&

(polje[current_index] ==1) &&

(cell_state[current_index] == CELL_OPENED) &&

cell_sprite_area &&

jedan[cell_sprite_y][cell_sprite_x];

wire cell_null = in_grid_area && (current_col < 8) && (current_row < 8) &&

(polje[current_index] == 0) &&

(cell_state[current_index] == CELL_OPENED) &&

cell_sprite_area &&

nula[cell_sprite_y][cell_sprite_x];

wire cell_mina = in_grid_area && (current_col < 8) && (current_row < 8) &&

(polje[current_index] ==9) &&

(cell_state[current_index] == CELL_OPENED) &&

cell_sprite_area &&

mine_rom[cell_sprite_y][cell_sprite_x];

wire cell_dva = in_grid_area && (current_col < 8) && (current_row < 8) &&

(polje[current_index] ==2) &&

(cell_state[current_index] == CELL_OPENED) &&

cell_sprite_area &&

dva[cell_sprite_y][cell_sprite_x];

wire cell_tri = in_grid_area && (current_col < 8) && (current_row < 8) &&

(polje[current_index] ==3) &&

(cell_state[current_index] == CELL_OPENED) &&

cell_sprite_area &&

three[cell_sprite_y][cell_sprite_x];

wire cell_four = in_grid_area && (current_col < 8) && (current_row < 8) &&

(polje[current_index] == 4) &&

(cell_state[current_index] == CELL_OPENED) &&

cell_sprite_area &&

four[cell_sprite_y][cell_sprite_x];

wire cell_five = in_grid_area && (current_col < 8) && (current_row < 8) &&

(polje[current_index] ==5) &&

(cell_state[current_index] == CELL_OPENED) &&

cell_sprite_area &&

five[cell_sprite_y][cell_sprite_x];

wire cell_six = in_grid_area && (current_col < 8) && (current_row < 8) &&

(polje[current_index] ==6) &&

(cell_state[current_index] == CELL_OPENED) &&

cell_sprite_area &&

six[cell_sprite_y][cell_sprite_x];

wire cell_seven = in_grid_area && (current_col < 8) && (current_row < 8) &&

(polje[current_index] == 7) &&

(cell_state[current_index] == CELL_OPENED) &&

cell_sprite_area &&

seven[cell_sprite_y][cell_sprite_x];

wire cell_eight = in_grid_area && (current_col < 8) && (current_row < 8) &&

(polje[current_index] == 8) &&

(cell_state[current_index] == CELL_OPENED) &&

cell_sprite_area &&

eight[cell_sprite_y][cell_sprite_x];

wire cell_flag = in_grid_area && (current_col < 8) && (current_row < 8) &&

grid_state2[current_index] &&

(cell_state[current_index] == CELL_FLAGGED) &&

cell_sprite_area &&

flag[cell_sprite_y][cell_sprite_x];

wire cell_unopened =	in_grid_area && (current_col < 8) && (current_row < 8) &&

(cell_state[current_index] == CELL_CLOSED) &&

cell_sprite_area &&

nula[cell_sprite_y][cell_sprite_x];

// Indeks trenutne æelije

// Ispravite kombiniranu logiku - koristite current_index umjesto clicked_cell

//////////////////////////////////

//wire cell_red = (current_col < 10) && (current_row < 10) && grid_state[current_row*10 + current_col];

assign VGA_HSYNC = hsync;

assign VGA_VSYNC = vsync;

// Glavna logika za VGA izlaze

////////////////////////////////////////////////////////////////9999999999999999999999999999999999999999999999

// ROM for digits 0-9 (8x8 pixels each) using your existing sprites

////////////////

parameter FLAG_TIMER_X = 265;

parameter FLAG_TIMER_Y = 153;

parameter DIGIT_WIDTH = 8;

wire [3:0]  tens, units;

wire tens_pixel, units_pixel;

wire timer_pixel;

assign timer_pixel = tens_pixel | units_pixel;

wire [4:0] timer_value = 10 - flag_count;

wire is_negative = (flag_count > 10);

wire [3:0] tens_digit =

is_negative ? 4'd10 :

(timer_value == 5'd10) ? 4'd1 : 4'd0;

wire [3:0] units_digit =

is_negative ? 4'd10 :

(timer_value == 5'd10) ? 4'd0 :

timer_value[3:0];

Flags tens_digit_inst(

.h_pos(h_counter - FLAG_TIMER_X),

.v_pos(v_counter - FLAG_TIMER_Y),

.digit(tens_digit),

.pixel(tens_pixel)

);

// 4. Instanciraj modul za jedinice

Flags units_digit_inst(

.h_pos(h_counter - (FLAG_TIMER_X + DIGIT_WIDTH)),

.v_pos(v_counter - FLAG_TIMER_Y),

.digit(units_digit),

.pixel(units_pixel)

);

wire tens_area =

(h_counter >= FLAG_TIMER_X) &&

(h_counter < FLAG_TIMER_X + DIGIT_WIDTH) &&

(v_counter >= FLAG_TIMER_Y) &&

(v_counter < FLAG_TIMER_Y + 16);  // Promijenjeno u 16 (visina fonta)

wire units_area =

(h_counter >= FLAG_TIMER_X + DIGIT_WIDTH) &&

(h_counter < FLAG_TIMER_X + 2*DIGIT_WIDTH) &&

(v_counter >= FLAG_TIMER_Y) &&

(v_counter < FLAG_TIMER_Y + 16);  // Promijenjeno u 16 (visina fonta)

// 6. Kombiniraj signale sa podruèjima

wire show_tens = tens_area && tens_pixel;

wire show_units = units_area && units_pixel;

wire show_timer = show_tens || show_units;

/*

wire [4:0] timer_value = 10 - flag_count;

wire is_negative = (flag_count > 10);

wire [3:0] tens_digit =

is_negative ? 4'd10 :          // "--" za negativno

(timer_value == 5'd10) ? 4'd1 : // 10 -> "1"

4'd0;

wire [3:0] units_digit =

is_negative ? 4'd10 :          // "--" za negativno

(timer_value == 5'd10) ? 4'd0 : // 10 -> "0"

timer_value[3:0];

wire tens_area =

(h_counter >= FLAG_TIMER_X) &&

(h_counter < FLAG_TIMER_X + DIGIT_WIDTH) &&

(v_counter >= FLAG_TIMER_Y) &&

(v_counter < FLAG_TIMER_Y + 8);

wire [2:0] tens_x = h_counter - FLAG_TIMER_X;

wire [2:0] tens_y = v_counter - FLAG_TIMER_Y;

wire tens_pixel = digit_rom[tens_digit][tens_y][tens_x];

wire show_tens = tens_area && tens_pixel;

// Jedinice (desno)

wire units_area =

(h_counter >= FLAG_TIMER_X + DIGIT_WIDTH) &&

(h_counter < FLAG_TIMER_X + 2*DIGIT_WIDTH) &&

(v_counter >= FLAG_TIMER_Y) &&

(v_counter < FLAG_TIMER_Y + 8);

wire [2:0] units_x = h_counter - (FLAG_TIMER_X + DIGIT_WIDTH);

wire [2:0] units_y = v_counter - FLAG_TIMER_Y;

wire units_pixel = digit_rom[units_digit][units_y][units_x];

wire show_units = units_area && units_pixel;

wire show_timer = show_tens || show_units;

*/

assign led_state = (flag_count >= 0) ? 10-flag_count : 255;

////////////////////////////////////////////////////////////////9999999999999999999999999999999999999999999999

// 5. Korektni VGA izlazi

assign VGA_R = (sprite_area && sprite_bit) ? 1'b1 :

(show_smiley) ? 1'b1 :

(show_smiley2) ? 1'b1 :

(show_FlagNum) ? 1'b1 :

(show_timer) ? 1'b1 :

(cell_mina && in_grid_area) ? 1'b1 :

(cell_flag && in_grid_area) ? 1'b1 :

(cell_eight && in_grid_area) ? 1'b1 :

(cell_seven && in_grid_area) ? 1'b1 :

(cell_six && in_grid_area) ? 1'b1 :

(cell_five && in_grid_area) ? 1'b1 :

(cell_four && in_grid_area) ? 1'b1 :

(cell_tri && in_grid_area) ? 1'b1 :

(cell_dva && in_grid_area) ? 1'b0 :

(cell_jedan && in_grid_area) ? 1'b0 :

(cell_null && in_grid_area) ? 1'b0 :

(cell_unopened && in_grid_area) ? 1'b1:

(grid && in_grid_area) ? 1'b1 : 1'b0;

assign VGA_G = (sprite_area && sprite_bit) ? 1'b1 :

(show_smiley) ? 1'b1 :

(show_smiley2) ? 1'b1 :

(show_FlagNum) ? 1'b0 :

(show_timer) ? 1'b1 :

(cell_mina && in_grid_area) ? 1'b1 :

(cell_flag && in_grid_area) ? 1'b0 :

(cell_eight && in_grid_area) ? 1'b0 :

(cell_seven && in_grid_area) ? 1'b0 :

(cell_six && in_grid_area) ? 1'b0 :

(cell_five && in_grid_area) ? 1'b0 :

(cell_four && in_grid_area) ? 1'b1 :

(cell_tri && in_grid_area) ? 1'b0:

(cell_dva && in_grid_area) ? 1'b1 :

(cell_jedan && in_grid_area) ? 1'b0 :

(cell_null && in_grid_area) ? 1'b0 :

(cell_unopened && in_grid_area) ? 1'b1:

(grid && in_grid_area) ? 1'b1 : 1'b0;

assign VGA_B = (sprite_area && sprite_bit) ? 1'b1 :

(show_smiley) ? 1'b0 :

(show_smiley2) ? 1'b0 :

(show_FlagNum) ? 1'b0 :

(show_timer) ? 1'b1 :

(cell_mina && in_grid_area) ? 1'b0 :

(cell_flag && in_grid_area) ? 1'b0 :

(cell_eight && in_grid_area) ? 1'b1 :

(cell_seven && in_grid_area) ? 1'b1 :

(cell_six && in_grid_area) ? 1'b1 :

(cell_five && in_grid_area) ? 1'b1 :

(cell_four && in_grid_area) ? 1'b0 :

(cell_tri && in_grid_area) ? 1'b0 :

(cell_dva && in_grid_area) ? 1'b0 :

(cell_jedan && in_grid_area) ? 1'b1 :

(cell_null && in_grid_area) ? 1'b0 :

(cell_unopened && in_grid_area) ? 1'b1:

(grid && in_grid_area) ? 1'b1 : 1'b0;

endmodule